	Model
	Quantity of Spindle
	Working Area
	Spindle Model
	Spindle Type
	Minimum Center Distance
	Capacity Max.
	Remarks

	CA-108
	2
	108
	Collet Chuck
	C-2 Fixed
	19
	φ6.5 for drilling
M8 for tapping
	

	
	
	
	
	C-2 Floating
	19
	M8
	for tapping only

	
	
	
	Jacobs Taper
	JT-1
	39
	φ6.5 
	Tool shank diameter

	CA-130
	2~4
	φ130
	Collet Chuck
	C-2 Fixed
	23
	φ6.5 for drilling
M8 for tapping
	

	
	
	
	
	C-2 Floating
	23
	M8 for tapping
	for tapping only

	
	
	
	
	C-3 Fixed
	26
	φ10.5 for drilling
M14 for tapping
	

	
	
	
	
	C-3 Floating
	26
	M14
	for tapping only

	
	
	
	Jacobs Taper
	JT-1
	39
	φ6.5 
	Tool shank diameter

	
	
	
	
	JT-2
	47
	φ10
	Tool shank diameter

	
	
	
	
	JT-6
	56
	φ13
	Tool shank diameter

	
	
	
	Adjustable Adaptor
	AS-16
	27
	φ16
	Tool shank diameter

	
	
	
	
	AS-22
	33
	φ22
	Tool shank diameter

	CA-170
	2~4
	φ170
	Collet Chuck
	C-3 Fixed
	29
	φ10.5 for drilling
M14 for tapping
	

	
	
	
	
	C-3 Floating
	29
	M14
	for tapping only

	
	
	
	
	C-4 Fixed
	32
	φ13 for drilling
M17 for tapping
	

	
	
	
	Jacobs Taper
	JT-1
	39
	φ6.5 
	Tool shank diameter

	
	
	
	
	JT-2
	47
	φ10
	Tool shank diameter

	
	
	
	
	JT-6
	56
	φ13
	Tool shank diameter

	
	
	
	Adjustable Adaptor
	AS-19
	33
	φ19
	Tool shank diameter

	
	
	
	
	AS-22
	33
	φ22
	Tool shank diameter

	CA-200
	2~4
	φ200
	Collet Chuck
	C-3 Fixed
	35
	φ10.5 for drilling
M14 for tapping
	

	
	
	
	
	C-3 Floating
	35
	M14
	for tapping only

	
	
	
	
	C-4 Fixed
	35
	φ13 for drilling
M17 for tapping
	

	
	
	
	Jacobs Taper
	JT-1
	39
	φ6.5 
	Tool shank diameter

	
	
	
	
	JT-2
	47
	φ10
	Tool shank diameter

	
	
	
	
	JT-6
	56
	φ13
	Tool shank diameter

	
	
	
	Adjustable Adaptor
	AS-16
	35
	φ16
	Tool shank diameter

	
	
	
	
	AS-19
	35
	φ19
	Tool shank diameter

	
	
	
	
	AS-22
	35
	φ22
	Tool shank diameter


